Assemble the Raspberry Pi

and Initialize Raspian Operating System

There are two ways to create an operating system (OS) for your Raspberry Pi.  Either OS creation must be done on a Windows computer. There is a simple system where you merely copy some files (NOOBS) onto the SD card and let the Pi itself download Raspbian the first time it is booted.  Alternately, you can downoad the full Raspbian OS and use an unZipper (7-Zip) and file-loader (win32diskimager) to download, unzip, and write the full Raspbian operating system (Raspbian) to the SD card.  If able, the latter is preferred.  The NOOBS derived version of Raspbian lacks some features of the full Raspbian system.

NOOBS (Easiest, but limited utility)

Get NOOBS Files

Create dir/sub/sub-directory called RaspberryPi/NOOBS/NOOBS-Extracted.”

Go to https://www.raspberrypi.org/downloads/noobs/.

Under “NOOBS Offline and network install,” click "Download ZIP."

Store the ZIP file in the directory: NOOBS.

Extract all files into directory: NOOBS-Extracted.

Create NOOBS SD Card

On main computer find directory: RaspberryPi/NOOBS/NOOBS-Extracted.

Copy all files onto a new or freshly formated (in FAT-32) micro-SD Card.

Raspbian (Preferred)

Gather Software Tools

From https://www.7-zip.org/download.html, download the appropriate 32/64-bit program.

From https://sourceforge.net/projects/win32diskimager/, download the win32diskimager.

Create Raspbian SD Card

On Windows computer create dir/sub-directory called RaspberryPi/Raspbian.”

Go to https://www.raspberrypi.org/downloads/raspbian/

Under “Raspbian Buster with desktop and recommended software,” click “Download ZIP.”

Using 7-Zip, unzip the Raspbian file to an image.

Insert SD card into reader/writer.

Write Raspbian image onto SD card.  This is a big file, so it will take a while.

Physical Assembly

Assure all AC power cables are disconnected from the monitor and power supply.

Insert the OS micro-SD Card into the Pi card slot.

Connect:	monitor HDMI cable, keyboard and mouse to Pi,

Ethernet cable to Pi and network, and

micro-USB power cable to the Pi and unpowered power supply.

Connect AC power cable to the monitor.

Switch the monitor ON.

Start the Pi

Connect the AC power to the power supply.  Pi should boot up.

If you are using NOOBS:

At the prompt, use the keyboard to select the “Raspbian” operating system.

Wait for the Raspbian operating system to load on the SD Card.

The Pi will boot Raspian and open the OS Configuration file.

Navigate with the keyboard arrow keys, to select (space bar selects/deselects) :

locale (eng_US.utf8) , and

SSH enable.

Close the configuration file.

If you need to return to this configuration file later, type:  sudo raspi-config

Select Keyboard

Edit the file:		sudo nano /etc/default/keyboard

Locate the line:	XKBLAYOUT=”gb”

Change the "gb" to "us".

Press ^o to save, and ^x to close the editor.

Then reboot:		sudo reboot

Change the Raspberry Pi's HostName

Edit the file:		sudo nano /etc/hosts

The very last entry, labeled 127.0.1.1, has the hostname "raspberrypi."

Replace "raspberrypi" with "rpi1  (or rpi2, rpi3, ...).

Press ^o to save, and ^x to close the editor.

Edit the file:		sudo nano /etc/hostname

This file only contains your hostname.

Replace “raspberrypi” with the same hostname used in the last step.

Press ^o to save, and ^x to close the editor.

Submit the changes:	sudo /etc/init.d/hostname.sh

Then reboot:		sudo reboot

Verify hostname:	hostname

Check router's device list to verify the new hostname.

Update Rasbian System Files to latest Revision

Revise package info:	sudo apt-get update

Download and upgrade s/w:

sudo apt-get upgrade

sudo reboot

Add a User

Type:			sudo adduser UU

Change The Password

Type:			sudo passwd UU

At the prompt, enter the new password.  It is effective immediately.

Add User to Groups (including making a Sudo-er)

Type:			usermod -a -G GG,GG,GG UU

example:	usermod -a -G sudo,pi user55

Type:			groups UU		(Verify by listing all groups for user UU.)

Set a Local Network Static IP-Address

(Alternately, a static IP can be set in the router.)

List the current network interface:

cat /etc/network/interfaces

The line, "iface eth0 inet dhcp", implies we get our IP Address via DHCP.

List the configuration file:

ifconfig

Look at the first couple of lines that look something like this:

eth0	Link encap:Ethernet HWaddr b8:27:eb:3f:9a:7d

inet addr:192.168.1.15  Bcast:192.168.1.255  Mask:255.255.255.0

Note the inet addr, Bcast, and Mask.

Type:			netstat -nr	... which will display:

--------------------------------------------------------------------------------------------------------

Destination	Gateway		Genmask	Flags	MSS Window	irll	Iface

0.0.0.0		192.168.1.1		0.0.0.0		UG	0 0		0	eth0

192.168.1.0	0.0.0.0			255.255.255.0	U	0 0		0	eth0

--------------------------------------------------------------------------------------------------------

Note the Gateway of 192.168.1.1, and the Destination of 192.168.1.0.

Edit network config:	sudo nano /etc/network/interfaces

Change line from:	iface eth0 inet dhcp

To:  			iface eth0 inet static

Add the lines:		address 192.168.1.15		(your "inet addr.", use your values.)

netmask 255.255.255.0	(your "Mask.")

network 192.168.1.0		(your "Destination.")

broadcast 192.168.1.255	(your "Bcast.")

gateway 192.168.1.1		(your "Gateway.")

Press ^o to save, and ^x to close the editor.

Remove exiting lease:	sudo rm /var/lib/dhcp/*

Reboot:			sudo reboot

Verify your work:		ifconfig

eth0	Link encap:Ethernet HWaddr b8:27:eb:3f:9a:7d

inet addr:192.168.1.15  Bcast:192.168.1.255  Mask:255.255.255.0

Ping your "Gateway" address" to verify:	ping 192.168.1.1 -c10

Change the SSH Server Port Number (but, why would you want to?)

Edit the file:		sudo nano /etc/ssh/sshd_config

Look for the line that reads:  "Port 22."

Change the port number to the desired port number.

Press ^o to save, and ^x to close the editor.

Reboot:		sudo reboot

When you now ssh into your Pi, you will need to mention that port number.

Example:	ssh -p 7322 pi@raspberrypi	(for port 7322)

To ssh access the Pi from the web, enable port forwarding on your router.

Change the Message Of The Day (MOTD)

This message will be displayed each time you log onto the Pi.

sudo nano /etc/motd

Edit to enter your desired message.

Press ^o to save, and ^x to close the editor.

Install gFortran

Revise package info:	sudo apt-get update

Install gfortran:	sudo apt-get install gfortran

Edit file:		nano .bashrc

Add line:	alias for='/usr/bin/gfortran -o'

Press ^o to save, and ^x to close the editor.

To compile Fortran program:

for fileOut fileIn.for

Install Apache

Revise package info:	sudo apt-get update

Install Apache:	sudo apt-get install apache2 -y

Test the web server.

If browsing from the same Pi:

http://localhost/

If browsing from another machine:

http://rpi3   (use actual hostname)

or	http://193.168.1.10   (use actual IP)

Observe the default test page, confirming Apache is working.

Note:	To verify the Pi's hostname on the LAN:

hostname

To find a Pi's IP on the LAN:

hostname -I

Install your own website HTML files:

Go to directory, observe the default index.html file, remove file:

cd /var/www/html

ls -al

sudo rm index.html

Copy your HTML files into this directory.

sudo curl -O http://www.spookmate.com/pi.htm

mv pi.htm index.html

If PHP file to be used, install PHP and the Apache PHP module:

sudo apt-get install php libapache2-mod-php -y

Create PHP to test:

Remove the index.html file, and create file index.php:

sudo rm index.html

sudo nano index.php

Put some PHP content in it:

Create a PHP file:

sudo nano index.php

Add one line:	<?php echo "hello world"; ?>

Press ^o to save, and ^x to close the editor.

Test the PHP web server:  Refresh your browser, and expect to see "hello world".

Remember:

HTML files go into the  /var/www/html/  directory, and

CGI files go into the  /usrlib/cgi-bin/  directory.

Factory settings:			User settings:

Username:  Pi				Username:  Pi

Password: raspberry			Password: (user selected)

Hostname: raspberrypi		Hostname: rpi1

SSH port #: 22			SSH port #: 22

Remote SSH Control

from another Linux Machine

Run the Pi Remotely via SSH

Run Pi remotely:	ssh pi@rpi1

At the prompt, enter the password.

The Pi's prompt is: “pi@rpi1 ~ $”

Temporarily escape the Pi to work on the local computer, and then return to the Pi

Escape the ssh process:

~^z		(an undisplayed tilde and then a control-z)

Your ssh process is pushed to the background, your terminal is now just back talking to the local computer.  The job number is displayed:

[#]+	Stopped		ssh pi@rpi1		(# is the job number.)

Remember the job number (#).

If you forget, find the job number:

jobs

[#]+	Stopped		ssh pi@rpi1		(# is the job number.)

Return to the Pi:	fg %#	Pulls ssh back to foreground (ex:  fg %1 ).

Exit the remote Pi

Type:			logout

You will be disconnected from the remote Pi, your ssh process terminated, and your termional is returned to the local system prompt.

Other Related Functions

Copy SD Card

To write an image of the entire uSD Card as a file on the Hard Drive of another computer, first assure to shutdown the Pi (procedure above).  Remove the uSD Card from the Pi, place it into a USB uSD Card reader, and place into another computer.  Determine the USB drive the uSD Card is installed into.  Use “pwd” to assure you are in the working directory where you want the file to be stored, then type:

dd if=DD of=FF		(Ex:  dd if=/dev/sdd of=backup.img)

This may take a while.

Create Backup Formated SD Card with Operating System

To write a uSD Card with a backup image, insert a new or newly formatted uSD Card into a USB uSD Card reader, and place into the computer with the backup image file on its hard drive.  (Assure you have this right, otherwise irreparable harm may come to this computer.)  Use “pwd” to assure you are in the working directory where the file exists, then type:

dd if=FF of=DD		(Ex:  dd if=backup.img of=/dev/sdd)

This may take a while.  Remove the USB uSD Card reader from the computer.  Remove the uSD Card from the reader.  Insert the uSD Card into the Pi.

Copy a File from/to the Local Computer to/from the Remote Pi

Copy from local computer to remote Pi:

scp FF pi@rpi1:/home/pi/FF

Copy from remote Pi to local computer:

scp pi@rpi1:/home/pi/FF FF

Start the X GUI     (Local command only)

Enter:			startx

SMTP Mail Setup

Load the packages:	sudo apt-get install ssmtp

sudo apt-get install mailutils

sudo apt-get install mpack

Edit file:		sudo nano /etc/ssmtp/ssmtp.conf

Edit the fields:		AuthUser=cgarybailey@gmail.com

AuthPass=password

FromLineOverride=YES

mailhub=smtp.gmail.com:587

UseSTARTTLS=YES

Press ^o to save, and ^x to close the editor.

To send email:		echo "Message text" | mail -s "Subject" username@domain.tld

To send attachments:	mpack -s "test" /home/pi/test/somefile.ext username@domain.tld

To text to Verizon:	echo "Message text" | mail -s "Subject" PhoneNum@vtext.com

Install a VNC on Pi

Install TightVNC:	sudo apt-get install tightvncserver

Run TightVNC	tightvncserver

It will prompt for a password and and optional view-only password.

Install a VNC Client on a Windows Machine

The TightVNC application is downloadable from www.Tightvnc.com.

Install a VNC Client on Local Linux Machine

Get application:	sudo apt-get install xtightvncviewer

Create a file:		sudo nano vnc.sh

Add script:	#! /bin/sh

vncserver :0 -geometry 1920x1080 -depth 24 -dpi 96

Press ^o to save, and ^x to close the editor.

Make the file executable:	chmod +x vnc.sh

Start TightVNC:		./vnc.sh

On the local computer, open the VNC client.  Enter the Pi's IP Address:port (0 or 1) and connect.

To end the session of the Pi:	killall vnc.sh